### 16 November 2000

Version 1.2

BOL CONTROL PLAN

For BOL MDT Chambers

## NIKHEF

| 3 | ASSEMBLY OF TUBE LAYER 1 (NAMING) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.1 | Lower spacer on towers in initial orientation, dismount crane bearings |  | Correct positioning of spheres on towers | Avoid damage to chamber and jigging |  |  |
| 3.2 |  |  | Check all monitor readings | Store in DB | Monitoring Manual |  |
| 3.3 | Compensate sag of cross plates with force (sag) compensation system for 0 tube layers |  | Pressure and force values | Store in DB | Monitoring Manual |  |
| 3.4 |  |  | Check all monitor readings, automatic warnings if out of range | Store in DB |  |  |
| 3.5 | Mount crane bearings, lift spacer |  |  | Avoid damage to chamber and jigging |  |  |
|  | INSERTION OF DRIFT TUBES INTO COMBS |  |  |  |  |  |
| 3.6 |  |  | Clean end combs with alcohol | Clean end combs |  |  |
| 3.7 | Insert drift tubes in combs in predefined sequence with vacuum off |  | Use gloves for all tube handling | Reliable bonding of the glue | QA/QC Manual |  |
| 3.7.1 | Take tubes out of the storage frame, one operator at each end, and insert them in the combs |  | Position barcode on top |  |  |  |
| 3.7.2 | During tube insertion in combs |  | Check for dust or defects on combs and tubes | Clean, repair, or remove if necessary | QA/QC Manual |  |
| 3.7.3 |  |  | Check if vacuum suction pads are damaged | Replace damaged suction pads | QA/QC Manual |  |
| 3.7.4 | Mount clocking pin on the north side of the tubes | Assembly Manual | Correct orientation of the ground pin mounting hole |  |  |  |
| 3.7.5 | Mount O-rings at the tube ends |  |  |  |  |  |
| 3.7.6 | Mount gas jumpers loosely, north side with bend up and south side with bend down | Assembly Manual | Check if gas jumpers are positioned correctly | Move gas jumpers to correct position, and/or correct orientation | Gas System Manual |  |
| 3.7.7 | Press tubes into the combs | Assembly Manual | Check orientation of the clocking pin at the north side and the ground-pin mounting hole at the south side | Avoid damage to tube and combs | QA/QC Manual |  |
| 3.7.8 | Tighten the gas jumpers |  | Correct positioning of tube end in longitudinal direction for leak tight gas connection |  |  |  |
| 3.7.9 | Lift the tubes up (per package of 3 tubes), ones from the north side and then ones from the south side |  | Correct positioning of tubes in the combs | Avoid damage to tube and combs |  |  |
| 3.7.10 | Switch on vacuum suction of the combs in defined sequence, starting at north side | Assembly Manual | Detect vacuum leaks by sound, check manometer reading (automatic warning if out of range) | Eliminate leaks, reinsert tubes, adjust vacuum, |  |  |
| 3.7.11 |  |  | Check if tubes are positioned correctly in longitudinal direction | Repeat step 3.7.9 if tubes are not touching orientation jig at north side |  |  |
| 3.7.12 | Remove gas jumpers and clocking pins |  |  |  |  |  |
| 3.7.13 |  |  |  | Store vacuum reading in DB |  |  |
|  | CONTROL OF TUBE PLACEMENT IN THE COMBS |  |  |  |  |  |
| 3.8. | Place drift tube barcode ID-strip on the tube end at north side |  |  |  |  |  |
| 3.8.1 | Start FOK-position program |  | Read tube barcode | Store in DB |  |  |
| 3.8.2 |  |  | Check for faulty drift tubes | Replace faulty drift tubes DB entry |  |  |
| 3.8.3 |  |  | Check gap between adjacent endplugs on combs |  |  |  |
| 3.8.4 |  |  | Check gap between adjacent tubes with illumination from below |  |  |  |
| 3.8.5 |  |  | Measure relative heights of endplug reference surfaces on combs with Guido laser, automatic warning if out of range | Reinsert or discard faulty tubes, check for dust or damage, store in DB |  |  |
| 3.8.6 |  |  | Automatic warning if vacuum is out of range If vacuum off: automatic restart within 10 seconds | DB entry Identify leak, adjust vacuum. If vacuum off: perform checks from checks 3.8.1 |  |  |
|  | GLUE DISTRIBUTION ON TUBE LAYER 1 |  |  |  |  |  |
| 3.9 |  |  | Mark location of glue ropes for gluing to spacer on tube layer |  |  |  |
| 3.10 | Glue distribution in gap between tubes with automatic glue dispenser | Gluing Manual | Uniform glue distribution with controlled size, location and start/end of glue ropes ensured by setting of glue dispenser, stop/start glue ropes before tube crimp | Minimal amounts of glue to minimise deformation due to shrinkage, no glue on combs | Gluing Manual |  |
| 3.10.1 | Setup glue dispenser | Gluing Manual | Check settings and positioning |  |  |  |
| 3.10.2 | Premix glue (Ciba Geigy Araldite 2011, fill glue dispenser reservoirs | Gluing Manual | Check glue type, quality, storage time and conditions Use immediately | Replace bad glue Store start time and temperature in DB | Gluing Manual |  |
| 3.10.3 | Start automatic glue distribution | Gluing Manual | Check quality of glue distribution continuously, visual inspection Glue dispenser runs only in presence of operator Stop after first glue line for inspection | Repairs with manual glue dispenser, keep ready in BOLroom | Gluing Manual |  |
| 3.10.4 |  |  | Automatic warning 10 minutes before maximum glue handling time (60 minutes) | Replace glue after maximum glue handling time exceeded, DB entry | Gluing Manual |  |
| 3.10.5 | End of glue distribution: dismount glue dispenser, clean in cleaning bath | Gluing Manual |  | Store time, temperature in DB | Gluing Manual |  |
|  | GLUING OF PRAXIAL ALIGNMENT PLATFORMS ON TUBE LAYER |  |  |  |  |  |
| 3.13 | Position praxial alignment platforms with templates on top of tube layer Fix distances between platforms in tube direction with aluminium rulers |  | Ensure correct placement of the platforms on the templates, of the templates on the endplug reference surfaces and tubes, and correct relative positioning of the templates |  |  |  |
| 3.13.1 | Fix platform to tube layerwith fast-curing glue (Ciba Geigy Araldite 2012), secure with 3M DP490 glue for long term |  |  |  |  |  |
| 3.13.2 | Dismount templates and aluminium rulers after curing of fast glue |  | Minimal glue curing time: 1 hour. Inspection of cured glue | Store end time of glue curing in DB |  |  |
| 3.13.3 |  |  | Measure relative angular alignment of the platforms and coplanarity with the granite table with dedicated optical monitors | Store readings in DB | Monitoring Manual |  |
|  | GLUING OF SPACER TO TUBE LAYER |  |  |  |  |  |
| 3.14 | Distribute glue beads (glue: 3M DP490) on tubes at marked position, with manual glue dispenser |  | Maximum glue handling time 90 minutes | Replace glue after maximum glue handling time exceeded | Gluing manual |  |
| 3.15 | Switch on the force compensation system |  | Force compensation engaged first to act instantaneously when spacer supported on towers |  |  |  |
| 3.16 | Lower spacer on towers in initial orientation, dismount crane bearings |  | Correct positioning of spheres on towers. | Avoid damage to chamber, combs, and towers |  |  |
| 3.16.1 |  |  | Check all monitor readings (temperature, optical, vacuum, force compensation) automatic warnings if out of range | Store in DB | Monitoring Manual |  |
|  | CURING OF GLUE` |  |  |  |  |  |